Date:06/10/2026

EXNO: 1 Estimation of Characteristic Impedance (Zo), and Propagation Constant (γ) of a Transmission Line from Primary Constants

Test Case 1: Investigating the significance of Characteristic Impedance (Zo), and Propagation Constant (γ) of a Transmission Line using the following parameters a) Resistance =10 Ω/Km b) Capacitance = 8 µF/Km c) Inductance =3.5 mH/Km d) Conductance =4 µmho/Km e) Frequency= 2 Khz

AIM: Investigating the significance of Characteristic Impedance (Zo), and Propagation Constant (γ) of a Transmission Line with Resistance =10 Ω/Km, Capacitance = 8 µF/Km, Inductance =3.5 mH/Km, Conductance =4 µmho/Km, Frequency= 2 Khz

SOFTWARE REQUIRED: SCILAB Version 6.1.1

PROCEDURE:

● Open SCILAB software

● Open file-new

● Put the input values

● Type the program

● Execute the program

● End the program

PROGRAM:

clear; clc;

R=10;L=0.003;f=2*10^3;G=4*(10^-6);C=16*(10^-6);

w=2*%pi*f;

Z=R+(%i*w*L);

Y=G+(%i*w*C);

Zo=sqrt(Z/Y);

C=round(real(Zo));

D=round(imag(Zo));

printf('-Zo = %f + j(%f)ohms\n',C,D);

P=sqrt(Z*Y);

a=real(P);

a1=round(a*10000)/10000;

printf('-Attenuation constant a = %f neper/km\n',a1);

b=imag(P);

b1=round(b*10000)/10000;

printf('-Phase constant b = %f radians/km',b1);

OUTPUT:

| Parameter | Simulation Results | Theoretical Results |
| --- | --- | --- |
| (Z_0) (Ω) | (21.05-j2.36) | (21.05-j2.36) |
| (alpha) (Np/km) | 0.2376 | 0.2376 |
| (beta) (rad/km) | 2.1161 | 2.1161 |

\

RESULT:

The characteristic impedance and propagation constant of a transmission line is calculated using SCILAB and the impact of primary constants on secondary constants are investigated.

Test Case 2: Examine the impact of resistance and inductance on the significance of Characteristic Impedance (Zo) and Propagation Constant (γ) of a Transmission Line at a frequency f=2.5 Khz. Vary the resistance value from 10 Ω to 50 Ω at the interval of 10 Ω with L,G and C values constant and calculate the Characteristic Impedance and Propagation Constant . Vary the inductance values from 3.5 mH to 11.5 mH at the interval of 2.5 mH with R,G and C values constant.

AIM: To examine the impact of resistance and inductance on the significance of Characteristic Impedance (Zo) and Propagation Constant (γ) of a Transmission Line at a frequency f=2.5 Khz. Vary the resistance value from 10 Ω to 50 Ω at the interval of 10 Ω with L,G and C values constant and calculate the Characteristic Impedance and Propagation Constant . Vary the inductance values from 3.5 mH to 11.5 mH at the interval of 2.5 mH with R,G and C values constant.

SOFTWARE REQUIRED: SCILAB Version 6.1.1

PROCEDURE:

● Open SCILAB software

● Open file-new

● Put the input values

● Type the program

● Execute the program

● End the program

PROGRAM:

clear; clc;

R=10;L=0.003;f=2*10^3;G=4*(10^-6);C=16*(10^-6);

w=2*%pi*f;

Z=R+(%i*w*L);

Y=G+(%i*w*C);

Zo=sqrt(Z/Y);

C=round(real(Zo));

D=round(imag(Zo));

printf('-Zo = %f + j(%f)ohms\n',C,D);

P=sqrt(Z*Y);

a=real(P);

a1=round(a*10000)/10000;

printf('-Attenuation constant a = %f neper/km\n',a1);

b=imag(P);

b1=round(b*10000)/10000;

printf('-Phase constant b = %f radians/km',b1);

OUTPUT:

| No. | R (Ω) | Simulation \(Z_0\) (Ω) | Simulation α (Np/km) | Simulation β (rad/km) | Theoretical α (Np/km) | Theoretical β (rad/km) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 19.53 − j2.55 | 0.2560 | 1.9635 | 0.2560 | 1.9635 |
| 2 | 20 | 19.99 − j4.97 | 0.5002 | 2.0100 | 0.5002 | 2.0100 |
| 3 | 30 | 20.67 − j7.22 | 0.7258 | 2.0777 | 0.7258 | 2.0777 |
| 4 | 40 | 21.47 − j9.27 | 0.9317 | 2.1582 | 0.9317 | 2.1582 |
| 5 | 50 | 22.34 − j11.13 | 1.1193 | 2.2455 | 1.1193 | 2.2455 |
| No. | L (mH) | Simulation \(Z_0\) (Ω) | Simulation α (Np/km) | Simulation β (rad/km) | Theoretical α (Np/km) | Theoretical β (rad/km) |
| 1 | 3.5 | 27.61 − j18.02 | 1.8113 | 2.7752 | 1.8113 | 2.7752 |
| 2 | 6.0 | 31.59 − j15.74 | 1.5829 | 3.1757 | 1.5829 | 3.1757 |
| 3 | 8.5 | 35.48 − j14.02 | 1.4092 | 3.5670 | 1.4092 | 3.5670 |
| 4 | 11.0 | 39.19 − j12.69 | 1.2758 | 3.9400 | 1.2758 | 3.9400 |

RESULT: The characteristic impedance and propagation constant of a transmission line is calculated using SCILAB and the impact of primary constants on secondary constants are investigated.

Test Case 3: Explore the impact of frequency and capacitance on the significance of Characteristic Impedance (Zo) and Propagation Constant (γ) of a Transmission Line. Vary the frequency from 2KHz to 10 KHz at the interval of 2 KHz with L,R ,G and C values constant and calculate the Characteristic Impedance and Propagation Constant . Vary the capacitance values from 8 µF to 16 µF at the interval of 2 µF with R,G and L values constant.

AIM: To explore the impact of frequency and capacitance on the significance of Characteristic Impedance (Zo) and Propagation Constant (γ) of a Transmission Line. Vary the frequency from 2KHz to 10 KHz at the interval of 2 KHz with L,R ,G and C values constant and calculate the Characteristic Impedance and Propagation Constant

PROGRAM:

clear; clc;

R=10;L=0.003;f=2*10^3;G=4*(10^-6);C=16*(10^-6);

w=2*%pi*f;

Z=R+(%i*w*L);

Y=G+(%i*w*C);

Zo=sqrt(Z/Y);

C=round(real(Zo));

D=round(imag(Zo));

printf('-Zo = %f + j(%f)ohms\n',C,D);

P=sqrt(Z*Y);

a=real(P);

a1=round(a*10000)/10000;

printf('-Attenuation constant a = %f neper/km\n',a1);

b=imag(P);

b1=round(b*10000)/10000;

printf('-Phase constant b = %f radians/km',b1);

OUTPUT:

| No. | \(f\) (Hz) | \(Z_0\) (Ω) | Simulation α (Np/km) | Simulation β (rad/km) | Theoretical α (Np/km) | Theoretical β (rad/km) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1000 | 19.994 − j4.974 | 0.2501 | 1.0050 | 0.2501 | 1.0050 |
| 2 | 1500 | 19.657 − j3.373 | 0.2544 | 1.4821 | 0.2544 | 1.4821 |
| 3 | 2000 | 19.532 − j2.546 | 0.2560 | 1.9635 | 0.2560 | 1.9635 |
| 4 | 2500 | 19.472 − j2.043 | 0.2568 | 2.4470 | 0.2568 | 2.4470 |
| 5 | 3000 | 19.440 − j1.705 | 0.2572 | 2.9315 | 0.2572 | 2.9315 |

| No. | \(C\) (F) | \(Z_0\) (Ω) | Simulation α (Np/km) | Simulation β (rad/km) | Theoretical α (Np/km) | Theoretical β (rad/km) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | \(8\times10^{-6}\) | 33.562 − j1.185 | 0.1490 | 4.2175 | 0.1490 | 4.2175 |
| 2 | \(10\times10^{-6}\) | 30.019 − j1.060 | 0.1666 | 4.7153 | 0.1666 | 4.7153 |
| 3 | \(12\times10^{-6}\) | 27.403 − j0.968 | 0.1825 | 5.1654 | 0.1825 | 5.1654 |
| 4 | \(14\times10^{-6}\) | 25.370 − j0.896 | 0.1971 | 5.5793 | 0.1971 | 5.5793 |
| 5 | \(16\times10^{-6}\) | 23.732 − j0.838 | 0.2107 | 5.9645 | 0.2107 | 5.9645 |

RESULT:

The impact of frequency and capacitance on the significance of Characteristic Impedance (Zo) and Propagation Constant (γ) of a Transmission Line is investigated for various values of frequencies and capacitance values.